EXHIBIT A

7.09.030.  General POTW Use Requirements.

- Prohibited Discharge Standards.  . . .

q.	Any discharge of fats, oils, or greases of animal or vegetable origin is limited to six one hundred (600) (100) mg/l.  . . .

7.09.040.  Pretreatment of Wastewater

- Separators and Hazards.  . . .

    - All existing commercial or industrial establishments shall have one (1) year one hundred eighty (180) days upon notification from the OCWRF to install a grease trap where required.  . . .

7.09.060.  Discharge Permit.

- Connections.

    - Any user proposing to begin or recommence discharging industrial wastes into the POTW, otherwise commonly known as the Waste Water Treatment Plant (WWTP), if the director has notified the industrial user of a violation of prohibited discharge standards as established in OMC 7.09.030 and that discharge has been suspended or the discharge permit revoked to stop the violation, the user, must obtain a wastewater permit prior to the beginning or recommencing of such discharge.  An application for this wastewater discharge permit shall must be filed at least ninety (90) days prior to the date upon which any discharge will begin or recommence.  The OCWRF has the right to place additional conditions or new or increased contributions of existing users.  . . .

- Modification. The director may modify the wastewater discharge permit for good cause including, but not limited to, the following:

    - A change in the POTW that requires either a temporary or permanent reduction or elimination of the authorized discharge. Information  , with findings indicating that the permitted discharge poses a threat to the OCWRF’s POTW, OCWRF personnel, or the receiving waters. . . .

7.09.170.  FATS, OILS, AND GREASE (FOG) CONTROL ORDINANCE

- Purpose and Intent

The purpose of this chapter is to protect the public sewer system from blockages, sanitary sewer overflows, backups, and treatment inefficiencies caused by fats, oils, and grease (FOG). This ordinance establishes requirements for grease interceptors or traps, best management practices, maintenance, and enforcement for users discharging wastewater to the City’s sanitary sewer system (POTW). Nothing in this chapter shall be construed to require the City to conduct routine inspections, monitoring, or reporting; enforcement actions may be undertaken by the City in response to observed conditions, complaints, sewer system impacts, or violations of this ordinance.

- Applicability

  - Any Food Service Establishment (FSE) and other non-residential users that generate or may generate FOG, are subject to this chapter, including but not limited to:

    - Restaurants, cafes, bars, or bakeries;

    - Grocery stores with food preparation areas;

    - Schools, churches, hospitals, or  care facilities;

    - Mobile food vendors; or

    - Any other user determined by the City to be a FOG contributor.

  - Residential users are exempt unless determined by the City to be a chronic source of sewer blockages.

  - The City’s determination of whether a user is a FOG Contributor shall be based on inspections, maintenance records, complaints, sewer system impacts, or other relevant information.

- Definitions

    - City means Oakley City, Utah.

    - FOG means fats, oils, and grease of animal or vegetable origin.

    - FOG Contributor means any user that, in the City’s determination, generates or may generate FOG that has the potential to adversely affect the public sewer system.

    - Food Service Establishment (FSE) means any facility engaged in preparing, serving, or selling food, including facilities with kitchens, dishwashing, or food preparation.

    - Grease Interceptor or grease trap means a gravity grease interceptor or other city-approved device intended to intercept and retain FOG prior to discharge to the public sewer system.

    - Hydromechanical Grease Interceptor (HGI) / Grease Trap means a hydromechanical device designed to separate and retain FOG and intended for indoor installation, unless otherwise approved by the City.

    - User means any non-residential person, business, or entity discharging wastewater to the POTW, excluding industrial user.

- Prohibited Discharges, Determinations and Findings

  - A user may not discharge or cause to be discharged to the public sewer system:

    - FOG in concentrations that cause or contribute to blockages, overflows, or interference with sewer operations;

    - Petroleum oils, mineral oils, or non-biodegradable greases;

    - Wastewater containing visible grease or free-floating oil;

    - Food waste, grease trap waste, or interceptor waste except by authorized disposal; or

    - Any discharge that, in the City’s determination, causes or contributes to interference with the operation or maintenance of the public sewer system.

  - Any discharge of fats, oils, or greases of animal or vegetable origin is limited to one  hundred (100) mg/l. Pollutants prohibited by this section shall not be processed or stored in such a manner that they could be discharged to the POTW. All floor drains located in process or materials storage areas must discharge to the users pretreatment facility before connecting with the POTW. If the user storing the specified pollutant does not have a pretreatment facility, the floor drain shall be either plugged with concrete or valved. The valve shall be locked closed at all times and opened only with permission from the director of the OCWRF.

  - The City shall determine compliance or noncompliance based on inspections, maintenance records, complaints, sampling, sewer system impacts, or other relevant information.

  - No user shall discharge FOGs to wastewater in the public sewer systems at a temperature exceeding 120ºF (49ºC). Wastewater shall not be discharged at any temperature that causes or contributes to FOG pass-through, deposition, or accumulation in the public sewer system.

  - The pollutant discharge standards that limit temperature of wastewater to 140ºF to the POTW shall govern, excluding FOG wastewater temperature limits as provided in this chapter.

- Design Submittals and Approval

All grease interceptors required by this ordinance shall be designed and sized by a professional engineer licensed in the State of Utah. The owner shall submit sealed design calculations, drawings, and specifications to the City for review and approval prior to issuance of a building permit or approval of a sewer connection. City approval is for conformance with this ordinance only and shall not be construed as acceptance of design responsibility or a guarantee of interceptor performance. No building permit, certificate of occupancy, or sewer connection approval shall be issued until such submittals are approved by the City.

- Grease Interceptor Trap Requirements

  - Installation

    - Any new commercial or industrial facility, including but not limited to FSEs, shall be required to install a properly sized grease interceptor unless the City determines, based on submitted engineering documentation, that FOG discharge will not occur.

    - Any existing FSE or other FOG Contributor may be required to install, replace, or upgrade grease interceptors if the City determines, based on inspections, maintenance records, complaints, sampling, sewer system impacts, or other relevant information that the facility contributes FOG-related blockages, backups, overflows, or interference with sewer operations.

    - Upon change of ownership or change of use of an existing facility, the owner shall demonstrate compliance with this ordinance as a condition of continued sewer service.

    - Hydromechanical grease interceptors or grease traps may be allowed only where specifically approved by the City based on the facility’s characteristics and demonstrated ability to prevent FOG discharge to the public sewer system.

  - Design and Sizing

    - Grease interceptors shall be designed and sized by a Utah-licensed professional engineer in accordance with the currently adopted plumbing code and accepted engineering practices.

    - Grease interceptors shall be located to provide safe and unobstructed access for inspection and maintenance and shall serve all grease-producing fixtures and drains, as applicable.

    - If feasible, grease interceptors should be located outside of the building served.

    - The City may reject designs that do not provide adequate access for maintenance or that are not reasonably expected to meet the intent of this ordinance.

    - The owner shall install an accessible sampling manhole, cleanout, or other approved structure downstream of grease control equipment to allow representative sampling of wastewater discharge for compliance with this ordinance.

- Operation and Maintenance

  - Operation and Maintenance Responsibility

    - The owner shall be solely responsible for the operation, maintenance, and performance of grease interceptors and associated plumbing. The City shall have no obligation to ensure ongoing compliance but may require increased cleaning frequency based on observed conditions.

  - Maintenance Frequency

    - The owner shall complete maintenance no less than every ninety (90) days, unless the City approves an alternative schedule in writing based on documented performance.

    - The owner shall increase cleaning frequency as needed based upon observed conditions and interceptor performance.

    - Grease interceptors shall be cleaned to prevent FOG carryover and sewer system impacts.

    - At no time shall combined grease and solids exceed 25 percent of the interceptor’s wetted volume.

  - Recordkeeping

    - Users shall maintain records for a minimum of three (3) years, including:

      - Date and time of cleaning;

      - Volume removed;

      - Disposal location;

      - Name and license number of hauler; and

      - Copies of disposal receipts or manifests, if applicable.

    - Records shall be maintained on site and made available to the City upon request.

- Best Management Practices (BMPs)

    - Any FSE shall implement BMPs, including:

      - Dry wiping cookware prior to washing;

      - Proper disposal of food scraps and grease;

      - Employee training on FOG control;

      - Posting of FOG BMP signage in food preparation areas;

      - No use of enzymes, emulsifiers, or additives as a substitute for required maintenance; and

      - No use of surfactants, enzymes, or other agents for the purpose of emulsifying or dissolving FOG to facilitate its discharge to the public sewer system.

    - The City may issue additional BMP requirements by policy without ordinance amendment.

- Inspections and Monitoring

    - As provided in OMC 7.09.010., the City has the authority of free access at all ordinary normal business hours, or immediately in the event of an emergency or imminent harm to the sewer system, to:

      - Enter facilities to inspect grease control equipment and examine the apparatus;

      - Review maintenance records; and

      - Conduct sewer line inspections or sampling upstream or downstream of a facility, including sampling and testing of discharge for compliance with city code.

    - Failure by the FSE to allow inspection constitutes a violation.

- Enforcement and Penalties

  - Notice of Violation. If the director finds that any user has violated or is violating this FOG ordinance or any wastewater discharge permit or order issued hereunder, the director or his agent shall issue a written Notice of Violation requiring corrective action and serve the Notice of Violation upon the user in person and documented as received by the user or by certified mail where a receipt is obtained.

  - User Response.  Within thirty (30) days of the receipt of the notice, the user shall submit an explanation to the director and a written plan for the satisfactory correction or prevention that includes specific required actions. Submission of the plan in no way relieves the user of liability for any violations occurring before or after receipt of Notice of Violation.

  - Compliance Order. If corrective action is not completed within thirty (30) days of receipt of the notice, the City may issue a Compliance Order requiring specific actions by a date certain, including installation or upgrade of grease control equipment, increased maintenance, implementation of BMPs, or other measures reasonably necessary to achieve compliance.

  - Penalties

    - Violations may result in:

      - Administrative fines;

      - Recovery of City costs for cleaning, repair, or overflow response; or

      - Increased inspection or maintenance requirements.

    - Each day of noncompliance constitutes a separate violation.

    - Separate violations may include failure to install required grease control equipment, failure to maintain equipment, failure to maintain records, or prohibited discharges.

  - Service Termination. The City may, at its discretion and after reasonable notice, suspend or terminate sewer service for:

- Repeated violations;

- Failure to install or maintain grease control equipment; or

- Discharge causing imminent harm to the sewer system or sewer employees.

10.6. The provisions of this chapter shall govern the discharge of FOGs. If any other provisions conflict with this chapter, this chapter shall supersede.

- Cost Recovery

Any user that is responsible for FOG-related blockage, backup, overflow, or other sewer system impact shall be liable for all associated City costs, including labor, equipment, contractor services, investigations, sewer cleaning, sampling and analytical testing, environmental response or remediation, or administrative costs.

- Appeals

Any user may appeal enforcement actions by submitting a written request to the City Recorder (or designated official) within ten (10) calendar days of the notice. The appeal shall be heard by the City Council or by a hearing officer or official designated by the City Council. Filing an appeal does not stay required corrective action unless approved in writing by the City.

- Procedural Rights

All users subject to this chapter and the City are governed by the procedural powers and protections as provided in OMC 7.09.110, Administrative Enforcement; 7.09.120, Judicial Enforcement; and 7.09.130, Supplemental Enforcement.